Glazing machine operation Manual

Thanks for purchasing the Glazing machine produced by our company. To ensure smooth operation and safe performance, please read the manual carefully before use:

1.  Installation and wire connection

1.1.Connect the 5-pin plug of the portable grinding head/hand piece to the OUTPUT socket at the main panel.

1.2.Connect the pedal switch to the FOO TS/W socket.

1.3.Select grinding mould of different sizes according to actual requirements, and fasten it into the front of the portable grinding head by turning the shaft lock.

1.4.Switch the speed control to the left.

1.5.Connect with the main source power.

2.  Operation

2.1.Press the function switch on the left to HAND for manual control, the indicator will be on and the grinding head starts to moving slowly. The diode LED1-2 will be on.

2.2.Turn the SPEED CONTROL to the right to accelerate the grinding head and all the LED indicators for the speed will be on.

2.3.When the function switch is pressed to FOOT, step on the pedal the speed will be accelerated, and loose the foot, the speed will be decreased(foot switch is optional).

2.4.The turning directions can be swapped: FWD means clockwise direction and REV means counterclockwise direction.

2.5.Adjust the speed of the grinding head within a reasonable range.

2.6.Please adjust the hand piece to the lowest speed before changing  forward/backward direction.

3.  Specifications

a) Main(power supply&control)part

1. Rated voltage range: 110-120V 60Hz

2. Rated power input: 15W

3. Output rated voltage: DC3-12V

4 .Output current: 0.8A

b) Hand piece (motor) part

1. Working voltage: DC3-12V

2. Rated current: 0.8A

3. Rated speed: 0rpm min-20,000/30,000 (customized available)rpm max

4.  lnstructions

4.1. Always unplug the tool when you change accessory or collet.

4.2. Do not change the turning direction too frequently; otherwise it will shorten the life of the grinding head.

4.3. Before changing forward/backward direction or before turn off the machine, please decrease the speed of the grinding head to the minimum.

4.4. Do not touch the bit after use for the bit is too hot to be touched by bare hands.

4.5. Do not engage the shaft lock button while the tool is running

4.6. Keep the machine out of water for it is not waterproof.

4.7. Keep dust out of eyes and lungs by using protective eyewear or a dust mask.

4.8. Always unplug it after use. When the machine is not in use, all tools must be stored in a dry location and keep out of reach of children.

4.9. Any unauthorized disassembling of the product is not suggested in case of any breakdown.